- ADC

  - Get ADC Value

| Function Name | ApiAdc_GetPedal1Vol |  |
| --- | --- | --- |
| Return Value | Uint16 | Get ADC Value for PIN ACC_PEDAL1_SIG |
| Notes | - Provide return value range 0~4095 (voltage range 0~5V) |  |

| Function Name | ApiAdc_GetPedal2Vol |  |
| --- | --- | --- |
| Return Value | Uint16 | Get ADC Value for PIN ACC_PEDAL2_SIG |
| Notes | - Provide return value range 0~4095 (voltage range 0~5V) |  |

- DIO

  - Get Pin Value

| Function Name | ApiDio_GetHoodSW |  |
| --- | --- | --- |
| Return Value | Uint8 | Get DI state for PIN HoodSw |
| Notes | The return back value should be 0/1 0: hood closed 1: hood opened |  |

| Function Name | ApiDio_GetTailgateSW |  |
| --- | --- | --- |
| Return Value | Uint8 | Get DI state for PIN TailgateSw |
| Notes | The return back value should be 0/1 0: Tailgate closed 1: Tailgate opened |  |

| Function Name | ApiDio_GetDrDoorSW |  |
| --- | --- | --- |
| Return Value | Uint8 | Get DI state for PIN DrDoorSW |
| Notes | The return back value should be 0/1 0: door closed 1: door opened |  |

| Function Name | ApiDio_GetPsDoorSW |  |
| --- | --- | --- |
| Return Value | Uint8 | Get DI state for PIN PsDoorSW |
| Notes | The return back value should be 0/1 0: door closed 1: door opened |  |

| Function Name | ApiDio_GetRRDoorSW |  |
| --- | --- | --- |
| Return Value | Uint8 | Get DI state for PIN RRDoorSW |
| Notes | The return back value should be 0/1 0: door closed 1: door opened |  |

| Function Name | ApiDio_GetRLDoorSW |  |
| --- | --- | --- |
| Return Value | Uint8 | Get DI state for PIN RLDoorSW |
| Notes | The return back value should be 0/1 0: door closed 1: door opened |  |

| Function Name | ApiDio_GetCentralDoorLockSW |  |
| --- | --- | --- |
| Return Value | Uint8 | Get DI state for PIN CentralDoorLockSW |
| Notes | The return back value should be 0/1 0: button released 1: button pressed |  |

| Function Name | ApiDio_GetHazardSW |  |
| --- | --- | --- |
| Return Value | Uint8 | Get DI state for PIN HazardSWSta |
| Notes | The return back value should be 0/1 0: button released 1: button pressed |  |

| Function Name | ApiDio_GetBrakeSW |  |
| --- | --- | --- |
| Return Value | Uint8 | Get DI state for PIN BRAKE_SW |
| Notes | The return back value should be 0/1 0: brake pedal released 1: brake pedal pressed |  |

| Function Name | ApiDio_GetBrakeLampSW |  |
| --- | --- | --- |
| Return Value | Uint8 | Get DI state for PIN Brake Stop Lamp |
| Notes | The return back value should be 0/1 0: brake pedal released 1: brake pedal pressed |  |

| Function Name | ApiDio_GetTrunkHanSW |  |
| --- | --- | --- |
| Return Value | Uint8 | Get DI state for PIN TrunkHanSW |
| Notes | The return back value should be 0/1 0: button released 1: button pressed |  |

| Function Name | ApiDio_GetSeatSW |  |
| --- | --- | --- |
| Return Value | Uint8 | Get DI state for PIN SeatSW |
| Notes | The return back value should be 0/1 0 = seat empty 1 = seat occupied |  |

| Function Name | ApiDio_GetCrashSignal |  |
| --- | --- | --- |
| Return Value | Uint8 | Get DI state for PIN VEHICLE_CRASH_SIG |
| Notes | The return back value should be 0/1 0 = GND 1 = B+ |  |

- PWM

  - PWM measurement

- System

## Get System Information

| Function Name | ApiSys_GetHWID |  |
| --- | --- | --- |
| Return Value | Uint8 |  |
| Notes | The return back value should be 0~255 |  |

- NM

## Set CAN NM Information

| Function Name | ApiCan_SetFrameRoutingEnable |  |
| --- | --- | --- |
| Set Value | Uint8 |  |
| Notes | All CAN Tx request from NM active=1 deactive=0 |  |

- APS

## Get PAS Value

| Function Name | ApiAps_GetDist |  |
| --- | --- | --- |
| Return Value | Void |  |
| Notes | Includes the following signals: APS_OUP_SNG_PAS_D_FCR APS_OUP_SNG_PAS_D_FMR APS_OUP_SNG_PAS_D_FML APS_OUP_SNG_PAS_D_FCL APS_OUP_SNG_PAS_D_RCR APS_OUP_SNG_PAS_D_RMR APS_OUP_SNG_PAS_D_RML APS_OUP_SNG_PAS_D_RCL APS_OUP_SNG_PAS_D_FLL APS_OUP_SNG_PAS_D_RLL APS_OUP_SNG_PAS_D_FLR APS_OUP_SNG_PAS_D_RLR |  |

| Function Name | ApiAps_GetDistLevel |  |
| --- | --- | --- |
| Return Value | Void |  |
| Notes | Includes the following signals: APS_OUP_U08_PAS_A_FCR APS_OUP_U08_PAS_A_FMR APS_OUP_U08_PAS_A_FML APS_OUP_U08_PAS_A_FCL APS_OUP_U08_PAS_A_RCR APS_OUP_U08_PAS_A_RMR APS_OUP_U08_PAS_A_RML APS_OUP_U08_PAS_A_RCL |  |

| Function Name | ApiAps_GetUssSta |  |
| --- | --- | --- |
| Return Value | Void |  |
| Notes | Includes the following signals: APS_OUP_U08_USS_Sta_FCR APS_OUP_U08_USS_Sta_FMR APS_OUP_U08_USS_Sta_FML APS_OUP_U08_USS_Sta_FCL APS_OUP_U08_USS_Sta_RCR APS_OUP_U08_USS_Sta_RMR APS_OUP_U08_USS_Sta_RML APS_OUP_U08_USS_Sta_RCL APS_OUP_U08_USS_Sta_FLL APS_OUP_U08_USS_Sta_RLL APS_OUP_U08_USS_Sta_FLR APS_OUP_U08_USS_Sta_RLR |  |

| Function Name | ApiAps_GetPASChime |  |
| --- | --- | --- |
| Return Value | Uint8 |  |
| Notes | The return back value should be 0~255 |  |

| Function Name | ApiAps_GetSensorLayout |  |
| --- | --- | --- |
| Return Value | Uint8 |  |
| Notes | The return back value should be 0~255 |  |

| Function Name | ApiAps_GetPASSystemFailSta |  |
| --- | --- | --- |
| Return Value | Uint8 |  |
| Notes | The return back value should be 0~255 |  |

| Function Name | ApiAps_GetPASSystemMode |  |
| --- | --- | --- |
| Return Value | Uint8 |  |
| Notes | The return back value should be 0~255 |  |

## Set PAS Information

| Function Name | ApiAps_SetVehSpeed |  |
| --- | --- | --- |
| Return Value | Single |  |
| Notes | Vehicle Speed Range:0~255.875 |  |

| Function Name | ApiAps_SetShiftGearPosn |  |
| --- | --- | --- |
| Return Value | Uint8 |  |
| Notes | Shift Gear Position Park:0 Neutral:4 Drive:5 Failure:6 Reverse:7 |  |

| Function Name | ApiAps_SetFrontSensorON |  |
| --- | --- | --- |
| Return Value | Uint8 |  |
| Notes | F_USS Switch Status OFF:0 ON:1 Invalid:2 |  |

| Function Name | ApiAps_SetVehSpeedTO |  |
| --- | --- | --- |
| Return Value | Uint8 |  |
| Notes | CAN message valid Valid=1 Invalid=0 |  |

| Function Name | ApiAps_SetFrontSensorSwTO |  |
| --- | --- | --- |
| Return Value | Uint8 |  |
| Notes | CAN message valid Valid=1 Invalid=0 |  |

| Function Name | ApiAps_SendChrgPlugSta |  |
| --- | --- | --- |
| Return Value | Uint8 |  |
| Notes | No plug =0 AC plug in =1 DC plug in =2 |  |

- SLEEP

  - Sleep command to BSP

# When this Api is called, BSP will start to sleep.

| Function Name | ApiSys_MCoreSleepRequest |  |
| --- | --- | --- |
| Set Value | Uint32 | state |
| Notes | (Currently no use) |  |
| Set Value | Uint32 | wakeup_timer_s |
| Notes | Sleep duration for BSP. After this time, FUSION should wake up. |  |
| Return Value | Uint8 |  |
| Notes | (Currently no use) |  |

  - NMM state transfer to BSP

| Function Name | ApiSys_SetCanNmState |  |
| --- | --- | --- |
| Set Value | Uint8 | state |
| Notes | NMM state enum definition: 0: SLEEP 1: REPEAT 2: NORMAL 3: READYSLEEP |  |

# 7.3 	SysPower state transfer to BSP

| Function Name | ApiSys_SetSysPwrStat |  |
| --- | --- | --- |
| Set Value | Uint8 | state |
| Notes | SysPower state enum definition: 0: OFF 1: OFF-C_HVChrg 2: OFF-A 3: STANDBY 4: ON_NoChrg 5: READY 6: Power_up 7: Power_down 8: HVChrgPre-action 9: OFF-C_LVChrg 10: ON_HVChrg |  |

# 7.4	Get A-core ready to sleep signal

| Function Name | ApiSys_GetACoreReadyToSleep |  |
| --- | --- | --- |
| Set Value | int8 | state |
| Notes | Signal value definition: 0: NOT READY 1: READY TO SLEEP |  |

# 7.5 Read and write flash function

| Function Name | ApiSys_ReadAppData |  |
| --- | --- | --- |
| Set Value | int8 | state |
| Notes | Signal value definition: 0: Read correct 1: Read fail |  |

| Function Name | ApiSys_WriteAppData |  |
| --- | --- | --- |
| Set Value | int8 | state |
| Notes | Signal value definition: 0: Read correct 1: Read fail |  |

- OTA

# 8.1	Get VINP_PwrReq_OTA_enum value

| Function Name | Apisys_GetVinpPwrReqOta |  |
| --- | --- | --- |
| Set Value | uint8 | state |
| Notes | Signal value definition: 0: No request 1: Request power ON 2: Request power OFF 3: Request power STANDBY 4: Invalid |  |

# 8.2	Get VINP_AllowMoveThd_OTA_enum value

| Function Name | Apisys_GetVinpAllowMoveThdOta |  |
| --- | --- | --- |
| Set Value | uint8 | state |
| Notes | Signal value definition: 0: Allow to READY 1: Not allow to READY 2: invalid |  |

- DID

# 9.1	Get DID_0201 for XWD mode Drivetrain

| Function Name | Apisys_GetDIDdata |  |
| --- | --- | --- |
| Set Value | uint8 | state |
| Notes | Signal value definition: 0: Read correct 1: Read fail |  |

- EVCC

# 10.1	Get VINP_EvccAplusPwrUpReq_flg value

| Function Name | Apisys_GetEvccAplusPwrUpReq |  |
| --- | --- | --- |
| Set Value | uint8 | state |
| Notes | Signal value definition: 0: No request 1: Request power up |  |